Note:

This document contains information and notes in hidden text.

If you need assistance to view & hide text, instructions are at this link:

http://www.marlite.com/hidden-notes-instructions.aspx

Delete this note and hyperlink after editing is complete.

The following specification has been developed by

Marlite

202 Harger St.

Dover, Ohio 44622

Tel: (330) 343-6621 Fax: (330) 343-4668.

Email:  info@marlite.com  www.marlite.com

to assist Architects and Designers in the specification of Decorative Fiberglass Reinforced Plastic wall panel system for high wear areas.

Marlite Stamdard FRP Panels are excellent for sanitary environments because they facilitate washing and cleaning and do not support mold or mildew growth.  However, FRP panels are subject to the effects of continuous humidity and temperature.  Provide adequate ventilation for drying and do not use in high temperature areas.

Marlite Induro FRP provides an exceptionally wear resistant decorative panel; due to an exclusive process of thermally bonding melamine impregnated surfacing materials directly to the FRP core.  Designed to match or coordinate with popular High Pressure Laminates, these panels offer a superb array of decorative finishes and patterns from solid colors to wood-grains and abstract patterns.  Custom designs and matching designs are available upon request

Marlite Artizan FRP panels offer a superb array of decorative printed rotogravure finishes with exceptional image integrity; unlimited varieties of woodgrain and abstract pattern plus tactile as well as visual textures.  Custom designs and matching designs are available upon request

Marlite Envue Graphic FRP provides impressive visuals of high-resolution photographs, illustrations, logos, tile designs and custom graphics, all protected by Marlite’s tough Sani-Coat sealer. A special application process produces optimum graphic reproduction on a tough, sanitary surface.

Marlite Symmetrix with Sani-Coat FRP panels feature prefinished precision grid score lines that provide the look of tile without grout to clean or deteriorate.  Custom designs and matching designs are available upon request.

The section has been formatted in accordance with the principals embodied in the Construction Specifications  Institute's  Manual of Practice and is intended for use in conjunction with Contract Conditions, Bidding Conditions and General Requirements developed in accordance with CSI guidelines.

Notes to the specifier are included in the boxes. Options that must be selected by the specifier are shown in brackets [         ].   Inapplicable clauses shall be deleted.

SECTION 09 7720 – DECORATIVE FIBERGLASS REINFORCED WALL PANELS

- GENERAL

      - SUMMARY

        - Section Includes:   Prefinished polyester glass reinforced plastic sheets and adhered to unfinished [gypsum] [cementitous] [untreated plywood] wallboard.

          - [Aluminum] [PVC] trim.

          - [PVC Wall base.]

        - Products Not Furnished or Installed under This Section:

          - Gypsum [Cementitious] substrate board.

          - Resilient Base.

      - RELATED SECTIONS

        - Section [____] – 	Gypsum [Cementitious] substrate board.

        - Section [____] - Wood [Metal] Stud Framing

        - Section [____] - Painting & Transparent Finishes.

        - Section [____] - Resilient Base.

      - REFERENCES

References are listed as a convenience and are often deleted in their entirety.  If required by the user agency delete references from the list below that are not actually required by the text of the edited section

        - American Society for Testing and Materials:  Standard Specifications (ASTM)

          - ASTM D 256 -	Izod Impact Strengths (ft #/in)

          - ASTM D 570 -	Water Absorption (%)

          - ASTM D 638 -	Tensile Strengths (psi) & Tensile Modulus (psi)

          - ASTM D 790 -	Flexural Strengths (psi) & Flexural Modulus (psi)

          - ASTM D 2583- Barcol Hardness

          - ASTM D 5319 - Standard Specification for Glass-Fiber Reinforced Polyester Wall and Ceiling Panels.

          - ASTM E 84 - Standard Test Method for Surface Burning Characteristics of Building Materials.

      - SUBMITTALS

        - Product Data:  Submit sufficient manufacturer's data to indicate compliance with these specifications, including:

          - Preparation instructions and recommendations.

          - Storage and handling requirements and recommendations.

          - Installation methods.

        - Shop Drawings:  Submit elevations of each wall showing location of paneling and trim members with respect to all discontinuities in the wall elevation.

        - Selection Samples:  Submit manufacturer’s standard color pattern selection samples representing manufacturer's full range of available colors and patterns.

        - Samples for Verification:   Submit appropriate section of panel for each finish selected indicating the color, texture, and pattern required.

          - Submit complete with specified applied finish.

          - For selected patterns show complete pattern repeat.

          - Exposed Molding and Trim:  Provide samples of each type, finish, and color.

        - Manufacturers Material Safety Data Sheets (MSDS) for adhesives and sealants prior to their delivery to the site.

      - QUALITY ASSURANCE

        - Conform to building code requirements for interior finish for smoke and flame spread requirements as tested in accordance with:

          - ASTM E 84 (Method of test for surface burning characteristics of building Materials)

            - Wall Required Rating – Class [A] [C].

- Include the following if products are used in areas subject to stringent wash down requirements.

        - Sanitary Standards:  System components and finishes to comply with:

          - United States Department of Agriculture (USDA) requirements for food preparation facilities, incidental contact.

          - Food and Drug Administration (FDA) 1999 Food Code 6-101.11.

          - Canadian Food Inspection Agency (CFIA) requirements.

      - DELIVERY, STORAGE AND HANDLING

        - Deliver materials factory packaged on strong pallets.

        - Store panels and trim lying flat, under cover and protected from the elements.  Allow panels to acclimate to room temperature (70°) for 48 hours prior to installation.

      - PROJECT CONDITIONS

        - Environmental Limitations:  Building are to be fully enclosed prior to installation with sufficient heat (70°) and ventilation consistent with good working conditions for finish work

        - During installation and for not less than 48 hours before, maintain an ambient temperature and relative humidity within limits required by type of adhesive used and recommendation of adhesive manufacturer.

          - Provide ventilation to disperse fumes during application of adhesive as recommended by the adhesive manufacturer.

      - WARRANTY

        - Furnish one year guarantee against defects in material and workmanship.

- PRODUCTS

      - ACCEPTABLE MANUFACTURER

        - Marlite;   202 Harger Street, Dover, OH  44622.  800-377-1221  FAX (330) 343-4668 Email:  info@marlite.com www.marlite.com.

        - Product:

          - Standard FRP

          - Induro FRP

          - Artizan FRP with Sani-Coat.

          - Envue with Sani-Coat.

          - Symmetrix with Sani-Coat.

      - PANELS

        - Fiberglass reinforced thermosetting polyester resin panel sheets complying with ASTM D 5319.

          - Coating:  Multi-layer print, primer and finish coats or applied over-layer.

          - Dimensions:

            - Thickness – 0.090 inch (2.29mm) nominal

            - Width - 4'-0”  (1.22m) nominal

            - Length – [10’-0” (3.0m)][8’-0” (2.4m) ][As indicated on the drawings] nominal

          - Tolerance:

            - Length and Width:  +/-1/8 inch (3.175mm)

            - Square -  Not to exceed 1/8 inch for 8 foot (2.4m) panels or 5/32 inch (3.96mm) for 10 foot (2.4m) panels

        - Properties:  Resistant to rot, corrosion, staining, denting, peeling, and splintering.

          - Flexural Strength - 1.0 x 104 psi per ASTM D 790. (7.0 kilogram-force/square millimeter)

          - Flexural Modulus - 3.1 x 105 psi per ASTM D 790. (217.9 kilogram-force/square millimeter)

          - Tensile Strength - 7.0 x 103 psi per ASTM D 638. (4.9 kilogram-force/square millimeter)

          - Tensile Modulus - 1.6 x 105 psi per ASTM D 638. (112.5 kilogram-force/square millimeter)

          - Water Absorption - 0.72% per ASTM D 570.

          - Barcol Hardness (scratch resistance) of 35 55 as per ASTM D 2583.

          - Izod Impact Strength of 72 ft. lbs./in ASTM D 256

        - Back Surface:  Smooth.  Imperfections which do not affect functional properties are not cause for rejection.

- Select finish from our full Marlite Finish Palette at www.marlite.com or contact Marlite to create your own Finish.

        - Front Finish: [As Indicated on the Drawings] [In accordance with preapproved sample.  Information available from the Architect’s Office][ __________________________________}

Specifier Note: Marlite’s Standard FRP, Induro FRP, Artizan FRP, Envue FRP & Symmetrix with Sani-Coat Panels are available in several configurations, including Class A (I) and Class C (III) Fire-rated, along with various surface textures for both Standard FRP & Symmetrix with Sani-Coat.  All Marlite FRP products can be made available with standard surface grooving and customized groove layouts to match architectural specifications.

        - Color: [Specifier to choose.]

              - Marlite Standard FRP is available in a variety of colors, including;

                - 100 White

                - 106 Beige,

                - 118 Natural Almond

                - 140 Ivory

                - 145 Silver

                - 151 Light Grey

                - P807 Black

                - 199 Bright White

- Custom patterns and colors are available.  Minimum quantities may apply.  Contact a Marlite representative for details.

              - Marlite Induro FRP is available in a variety of colors and patterns, including;

                - D354 Designer White

                - D381 Fashion Grey

                - 4143 Neutral Glace

                - 4746 Woolamai Brush

                - FP610 Hard Rock Maple

                - FP612 Graphite Spectrum

                - 7061 Natural Pear

                - 7922 Brighton Walnut

                - 7924 Biltmore Cherry

                - 7925 Monticello Maple

- Custom patterns and colors are available.  Minimum quantities may apply.  Contact a Marlite representative for details.

              - Marlite Artizan FRP is available in a variety of colors and patterns, including;

                - 148 Monticello Anigre

                - 310 Sierra Maple

                - 715 Mahogany

                - 925 Autumn Cherry

                - 5408 Monterey Sand

                - 5409 Townsend

                - 5410 Sebring

                - 5411 Rainier

                - 194 Cody

                - 192 Laramie

                - 209 Adara

                - 211 Grizel

                - 181-G1212 Dusk

                - 182-G1212 Oxide

                - 5415-G88 Coronado

                - 5416-G88 Catalina

                - T936-G44 Milan

                - T938-G44 Tuscany

                - T938-G88 Tuscany

                - T939-G44 Verona

                - T939-G88 Verona

                - T5412-G1212 Palermo

                - T5413-G1212 Genoa

                - T5414-G88 Modena

                - T5417-G88 Salerno

                - T5418-G88 Tangier

                - T5419-G88 Madras

                - T5420-G88 Ceylon

- Custom patterns and colors are available.  Minimum quantities may apply.  Contact a Marlite representative for details.

              - Marlite Envue FRP includes full custom graphics capabilities and are provided as;

                - Customer supplied artwork, in a digital file format compatible with Marlite’s requirements and in a layout compatible with the panel dimensions as required for the installation.  Revisions by Marlite Graphics Services to customer supplied artwork are to be submitted for customer approval.  It is the customer’s responsibility to contact their Marlite representative for Marlite’s digital file format requirements. Charges may apply if revisions are required to re-format artwork files.

                - Marlite supplied artwork, for customer review and approval.  Graphic serviced and set-up charges may apply.

              - Marlite Symmetrix with Sani-Coat FRP is available in a variety of colors, patterns and groove configurations, including;

                - C100-G44 White

                - C100-G66 White

                - C118-G44 Almond

                - C140-G44 Ivory

                - C145-G44 Silver

                - C105-G44 Black

                - C131-G44 Everglade

                - C133-G44 Periwinkle

                - C150-G44 White with Black

                - C151-G44 White with Everglade

                - C153-G44 White with Periwinkle

- Custom patterns and colors are available.  Minimum quantities may apply.  Contact a Marlite representative for details.

        - Surface [Specifier to choose.] Marlite Standard FRP and Symmetrics are available on two standard surface textures. Pebbled (indicated by “P” designation i.e., P 100 White) and Smooth (indicated by “S” designation i.e., S 100 White).  All other Marlite FRP products are smooth surface texture.

        - Fire Rating [Specifier to choose.] Marlite FRP is available in either Class A (I) or Class C (III) Fire Rating.  Specifier should add designation to product number only for Class A (i.e., P 100 White, Class A).

        - Size: [Specifier to choose, or as indicated on drawings.] Marlite FRP standard sizes are;

          - 4’ x 8’ [1.2m x 2.4m] x .120”

          - 4’ x 9’ [1.2m x 2.7m] x .120”

          - 4’ x 10’ [1.2m x 3m] x .120”

          - Ceiling panels (designed for suspended ceiling installation)

- 23 ¾” x 47 ¾” (0.6m x 1.2m) x .120”

- 23 ¾” x 23 ¾” (0.6m x 0.6m) x .120”

- Can be specified as FRP panels only or backed with 5/8” FR Drywall.

- Delete the following if resilient bases are used

      - BASE

        - Marlite Base Molding for 0.090 inch (2.29mm) thick FRP Panels

          - Color: [Black] [Quarry Red].

          - Profiles:

            - M 612 FRP Base Molding

            - M 651 Inside Corner

            - M 660 Outside Corner

            - M 620 LH End Cap

            - M 625 RH End Cap

      - MOLDINGS

- Select PVC or Aluminum trim for Envue and Symmetrix systems.  PVC trims are not available on Artisan.

        - Aluminum Trim:  Heavy weight extruded aluminum 6063-T5 alloy prefinished at the factory.

          - Finish:  Factory oven-baked finish.

          - Profiles	:

- The following are Symmetrix and Envue trim profiles.

            - F 550 Inside Corner

            - F 561 Outside Corner

            - F 565 Division

            - F 570 Edge

- The following are Artisan trim profiles.

            - A551 Inside Corner.

            - A560 Outside Corner

            - A565 Division

            - A570 Edge

          - Color:  [Bright Anodized] _____________

- PVC colors are available to coordinate with Sani-Coat colorways.  See current Marlite Finish Guide for available choice.

        - PVC:  Extruded PVC Trim Profiles for .090 inch thick panels.

          - M 350 Inside Corner

          - M 360 Outside Corner

          - M 365 Division

          - M 370 Edge

          - Color:  ________________

        - Outside Corner Guard:

          - F 560 Stainless Steel

            - Finish:  #4 satin

          - M 961 PVC

            - 199 White

            - 118 Natural Almond

            - 140 Ivory

            - 145 Silver

            - 151 Light Grey

      - ACCESSORIES

- Specify rivets for use in areas with dramatic changes in temperature and/or humidity, where the substrate is unusually uneven, and in all low temperatures or cold storage applications.

        - Fasteners:  Non-staining nylon drive rivets.

          - Match panel colors.

          - Length to suit project conditions.

        - Adhesive:  Either of the following construction adhesives complying with ASTM C 557.

          - Marlite C-551 FRP Adhesive  - Water- resistant, non-flammable adhesive

          - Marlite C-375 Construction adhesive flexible, water-resistant, solvent based adhesive formulated for fast, easy application.

        - Sealant:

          - Marlite Brand MS-250 Clear Silicone Sealant

          - Marlite Brand MS-251 White Silicone Sealant

- Colors available to coordinate with Marlite Artizan colorways..

          - Marlite Brand - Color Match Sealant,

- EXECUTION

      - PREPARATION

        - Examine backup surfaces to determine that corners are plumb and straight, surfaces are smooth, uniform, clean and free from foreign matter, nails countersunk, joints and cracks filled flush and smooth with the adjoining surface.

          - Verify that stud spacing does not exceed 24 inch (61cm) on-center.

        - Repair defects prior to installation.

          - Level wall surfaces to panel manufacturer’s requirements.  Remove protrusions and fill indentations.

      - INSTALLATION

        - Comply with manufacturer's recommended procedures and installation sequence.

        - Cut sheets to meet supports allowing 1/8” inch (3 mm) clearance for every 8 foot (2.43m) of panel.

          - Cut and drill with carbide tipped saw blades or drill bits, or cut with shears.

          - Pre-drill fastener holes 1/8 inch (3.175mm) oversize with high speed drill bit.

            - Space at 8 inches (20.32cm) maximum on center at perimeter, approximately 1 inch from panel edge.

            - Space at in field in rows 16 inches (40.64cm) on center, with fasteners spaced at 12 inches (30.48 cm) maximum on center.

        - Apply panels to board substrate, above base, vertically oriented with seams plumb and pattern aligned with adjoining panels.

          - Install panels with manufacturer's recommended gap for panel field and corner joints.

            - Adhesive trowel and application method to conform to adhesive manufacturer’s recommendations.

            - Drive fasteners for snug fit.  Do not over-tighten.

        - Apply panel moldings to all panel edges using silicone sealant providing for required clearances.

          - All moldings must provide for a minimum 1/8 inch (3.18mm) of panel expansion at joints and edges, to insure proper installation.

          - Apply sealant to all moldings, channels and joints between the system and different materials to assure watertight installation.

      - CLEANING

        - Remove excess sealant from panels and moldings.  Wipe panel down using a damp cloth and mild soap solution or cleaner.

        - Refer to manufacturer's specific cleaning recommendations Do not use abrasive cleaners.

END OF SECTION 09 7720