PART 1 GENERAL

## 1.1	SECTION INCLUDES

- Pre-finished Slatwall slatwall panel system for merchandise displays.  Use as perimeter walls and/or floor fixtures for Retail displays.

  - RELATED SECTIONS

- Section 06420 – Paneling:  Architectural paneling.

- Section 09771 – Decorative Wall Surfacing:  Manufactured paneling.

  - SUBMITTALS

- Product Data:  Manufacturer’s product data sheets on each product to be used.

- Product Drawings:  Manufacturer’s drawings showing details and dimensions.

- Product Installation:  Manufacturer’s installation instruction & procedures.

- Product Samples:  For each finish specified, select from manufacturer’s full range of available colors and patterns.

  - DELIVERY, STORAGE, AND HANDLING

- Open packaging promptly after deliver and inspect panels carefully.  Contact Freight carrier if damaged in transit.  Contact manufacturer for replacement order.

- Store panels in cool dry environment.  Do not subject to moisture.

- Do not stack panels directly on floor.

  - PROJECT CONDITIONS

- Condition panels to normal room temperature, and low humidity prior to installation

  - WARRANTY

- All products shall be warranted to be free of defects for a period of 30 days after installation.

PART 2 PRODUCTS

  - MANUFACTURER

A.	Slatwall slatwall, Display panels shall be Manufactured by:  Marlite, 202 Harger Street, Dover, OH 44662.  ASD. Tel:  330-343-6621. Fax:  330-343-7296.  Email:  info@marlite.com	Web:  www.marlite.com

  - MATERIALS

- All pre-finished panels & floor fixtures shall be Slatwall slatwall panels.

- All Slatwall slatwall panels shall be constructed utilizing a 48# density medium density fiberboard (MDF) substrate, having an internal bond strength of 110# per square inch minimum.  All Slatwall slatwall panels shall have formaldehyde emissions of .3 PPM or less and shall comply with HUD 24 CFR Part 3280 Standards set fourth for particleboard panels.

- All Slatwall slatwall panels shall have engineered “T” grooves factory machined as follows (specifier to select one or more :)

      - 2000 Series: engineered groove machined into ¾” thick substrate.

        - Optional groove finish may be specified:

          - Painted groove:  available on panels surfaced with Marlite’s exclusive finish.

          - PVC solid color trim trips:  specifier to select from manufacturer’s color options.

          - ColorSnaps (patented) snap-in inserts:  specifier to select from manufacturer’s color options.

          - HPL trim strips:  specifier to select from HPL manufacturer’s color options.

      - 5000 Series:  engineered groove machined into ¾” thick substrate, reinforced with a patented PVC full grove insert.  Insert finish color to be selected from manufacturer’s color options.  Inserts are to be factory installed.

      - 3000 Series:  engineered groove machined into ¾” thick substrate, reinforced with a patented aluminum L-groove insert.  Insert finish color to be selected from manufacturer’s color options.  Inserts are to be factory installed.

      - 7000 Series:  engineered groove machined into ¾” thick substrate, reinforced with a patented aluminum T-groove insert.  Insert finish color to be selected from manufacturer’s color options.  Inserts are to be factory installed.

- All Slatwall slatwall panels shall have surface (face) finish as follows (specifier to select from manufacturer’s finish guide :)

      - Marlite’s finish:  a modified melamine enamel that is baked to achieve outstanding performance characteristics.

      - LPM (Low Pressure Melamine) Melamine treated paper bonded to the substrate via hot melt adhesive.

      - High Pressure Laminate:  a .030” thick high-pressure laminate sheet bonded via cold press with polyvinyl acetate (PVA) type II, water resistant adhesives.  Finish color to be selected from HPL manufacturer’s color options.

      - Wood Veneers:  an architectural grade rotary cut veneer (.035” thick) applied to the substrate via cold press with balancing veneer backer sheet.  Custom veneer and grades available.

      - Mirrored Acrylic:  a .080” thick acrylic sheet backed with reflective film, cold press laminated to the substrate.

      - Primed Finish:  an alkyd primer is factory applied and baked creating a suitable surface for field finish in one coat.

      - Paint Grade:  a smooth unfinished surface suitable for field or in-shop finishing.

- All Slatwall slatwall panels shall have grooves machined on (specifier to select :)

      - 3” on-center

      - 6” on-center

      - 12” on-center

      - Other (specify)

  - FABRICATION / FACTORY FINISHING

- Tolerances for panels:

      - Dimensional:   .0625”

      - Squareness:  .125” across diagonals

      - Thickness:   .008”

      - Grooving::   .031” (groove width and spacing between grooves)

PART 3 EXECUTION

  - PREPARATION

- All walls or partitions which are to receive Slatwall slatwall panels shall be dry, solid and flat.  Recommended stud spacing is 16” on-center.  Slatwall slatwall panels may be applied directly to open studs or over drywall.  Care must be taken to prevent moisture penetration through the walls.  Refer to manufacturer’s installation instructions.

  - INSTALLATION

- All Slatwall slatwall panels are to be installed in strict accordance with the manufacturer’s written installation instructions.